Lecture 26

# Issues in Factory Design and Construction

	A fab is in many respects a chemical plant.

* costs $2 billion

* takes ~2-3 years to build

* process time is several weeks from start to finish

* throughput now at 20K wafer starts/month (WSM)

* in near future, throughput will be 30-40K WSM

* process steps will double in 10 yrs

	Recent hiring data from Intel and IBM indicates that a typical fab hires 25-30% ChE’s – fraction not exceeded by any other discipline.

## Some context for fab construction: comparative industry features

	Bulk petrochemicals

Example:  BP-Amoco

ROI goal ~6%, was 3% in mid-90s

Key features:	* debottlenecking, optimization are crucial

* few process innovations

* profitability significantly impacted by mergers

	Consumer products

Example: Kraft:

ROI ~ 8%, was 3% a decade or so ago

Key features:  	* short product life cycle

* broad arrays of new products, heavily marketed

* technical sophistication often modest compared to

electronic device processing

	Microelectronics

Example: Intel

ROI in double digits

Key features:	* high technological sophistication

* rapid introduction of improved technologies

* for microprocessors, price remains roughly flat

while performance increases according to

steep power law

* symbiotic relationship with software industry

Main point: Combination of rapid technology advance and high capital costs require detailed planning and coordination at a variety of levels.

### See Handout, first page

Note: 	 The “zero” year is usually defined as the time at which the

first million units have been shipped.

 There are well-defined timelines for a variety of activities

 There are significant overlaps (and coordination) among the

various activities.

## Fab-level construction issues

Siting Issues:

- access to good workforce (e.g., problems at Micron in Boise)

- government incentives

- environmental regulations

- access to water

- proximity to other parts of industry (San Jose, Research Triangle Park, Boston Rt. 301, Austin)

- cost of land

Design issues:

- ability to add/modify for future generations

- keep chemical handling (esp. gases) separate (~25% of space)

- const. T in critical areas (T ~1C)

- anti vibration for lithography (~20% of building cost)

- air handling in clean rooms

## Optimizing plant assets

# A. Tools

- Tools used to be 30% of fab cost.  Now >80%.

- Implanters, steppers, furnaces, etc, are $100K to $10M apiece.

- Requires lots of environmentally controlled floor space

- Yield levels are now near 90% – can’t improve much more.  More attention to debottlenecking

e.g.  Equipment is used only 30% of the time in production!

Unsch. shutdown 

 cancelled orders

 late materials

 operators quit

 software failures

(80% of total

shutdowns in

new equipment)

B. Consumables

- Substrate wafers

- Process chemicals

Minimizing costs requires that inventory levels be optimized:

- too much  excessive floor space, expiration of products with limited shelf life

- too little  shortages

C. Subcontractor relationships

Subcontracted services (due to high cost-of-ownership, COO)

 wafer growth

 assembly

 electrical test

 die preparation

 final test

 packaging

 sometimes implant (complex)

- must deliver promised work to subcontractor

- not put in erratic requirements

- include required paperwork

D. Data management

Increases productivity of process engineer  don’t want to spend time getting data

Types:		standard  (good at job level: yield variance)

	ad hoc  (equipment/process specific)

	analysis  (piece of equipment or entire shift)

Software exists to help with this (manufacturing execution system)

Lecture End ––––