Previously Logo’d Motherboard Logo Program (PLMP)

Intel® Desktop Board

DQ67SW

PLMP Report

7/31/2026

Purpose:

This report describes the DQ67SW Previously Logo’d Motherboard Logo Program testing run conducted by Intel Corporation.

THIS TEST REPORT IS PROVIDED "AS IS" WITH NO WARRANTIES WHATSOEVER, INCLUDING ANY WARRANTY OF MERCHANTABILITY, FITNESS FOR ANY PARTICULAR PURPOSE, OR ANY WARRANTY OTHERWISE ARISING OUT OF ANY PROPOSAL, SPECIFICATION OR SAMPLE.

Information in this document is provided in connection with Intel products. No license, express or implied, by estoppels or otherwise, to any intellectual property rights is granted by this document or by the sale of Intel products. Except as provided in Intel’s Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel products are not intended for use in medical, life saving, or life sustaining applications.

Intel retains the right to make changes to its test specifications at any time, without notice.

The hardware vendor remains solely responsible for the design, sale and functionality of its product, including any liability arising from product infringement or product warranty.

Intel® Core™ i7 and Intel® are trademarks of Intel Corporation in the U.S. and other countries.

*Other names and brands may be claimed as the property of others

Copyright © 2009, Intel Corporation. All rights reserved.

Contents

Introduction	4

Terms and Definitions	4

Desktop Board Configuration	5

Desktop Board DQ67SW Final Configuration Report: Completion of SLP	5

Board Information	5

Product Code	5

Processor	5

Motherboard	5

System Memory	5

Power Management	5

Operating System Tested	5

Onboard Integrated Devices and Driver for Windows 7 32-bit and 64-bit	6

Additional Devices and Driver for Windows 7 32-bit and 64-bit	7

Windows Logo Kits Used (WLK)	8

Errata and Contingencies	9

Test Notes	12

# Introduction

## Terms and Definitions

| Term | Definitions |
| --- | --- |
| WHQL | Windows* Hardware Qualification Lab |
| WLK | Windows Logo Kits |
| PLMP | Previously Logo’d Motherboard Logo Program. For further information see: http://www.microsoft.com/whdc/hwtest/default.mspx |
| AP Machine | Audio Precision Machine |
| Winqual | Windows Qualification |
| MSFT Tested Product List | Tested Products List. You can view the Windows Marketplace for tested products list at: http://winqual.microsoft.com/HCL/ProductList.aspx?m=v&cid=105&g=s |

# Desktop Board Configuration

## Desktop Board DQ67SW Final Configuration Report: Completion of PLMP

Data in this section reflects system configuration at time of PLMP submission.

### Board Information

| Product Code | BIOS String/Model | Technologies NOT Logo’d (yet) |  |
| --- | --- | --- | --- |
| DQ67SW | SWQ6710H.86A.0032.2010.1204.0539 | N/A - all technologies logo’d |  |
| Processor |  |  |  |
| Speed | 3.4GHz |  |  |
| Family | Intel® Corei7-2600 D2 |  |  |
| Bus Speed | 100 MHz |  |  |
| Motherboard |  |  |  |
| Board AA # | G12527 |  |  |
| Board FAB # | 300 |  |  |
| * This report applies to the production FAB revision; Please consult your Intel Corporation representative to clarify the motherboard revision you intend to perform logo testing if not the same. |  |  |  |
| System Memory |  |  |  |
| Speed | Dual Channel, DDR3, 1333MHz |  |  |
| Memory Type | DIMM |  |  |
| Connector Type | DDR3, 240 Pin |  |  |
| Power Management |  |  |  |
| BIOS Default | S3 |  |  |
| Operating System Tested |  |  |  |
|  | Check Tested |  | Comments |
| Windows 7 32 and 64-bit |  |  | Windows 7 Ultimate |

### Onboard Integrated Devices and Driver for Windows 7 32-bit and 64-bit

| Technology | OS | Version | Package version |
| --- | --- | --- | --- |
| Chipset Update Utility Intel® Chipset Software Utility | Windows 7. Windows 7 64-bit. | 9.2.0.1015. 9.2.0.1015. | 9.2.0.1015. 9.2.0.1015. |
| On-board GFX Intel(R) HD Graphics Family | Windows 7. Windows 7 64-bit.. | 8.15.10.2246. 8.15.10.2246. | 15.21.2.2246. 15.21.2.64.2246. |
| Audio Realtek High Definition Audio | Windows 7. Windows 7 64-bit.. | 6.0.1.6235. 6.0.1.6235. | PG313. PG313. |
| On-board Audio Realtek High Definition Audio | Windows 7. Windows 7 64-bit. | 6.14.0.3074. 6.14.0.3074. | 15.21.2.2246. 15.21.2.64.2246. |
| LAN Intel 82579LM Gigabit Network Connection | Windows 7. Windows 7 64-bit. | 11.8.81.0. 11.8.81.0. | N/A. N/A. |
| USB 3.0 Renesas Electronics USB3.0 | Windows 7. Windows 7 64-bit. | 2.0.26.0. 2.0.26.0. | N/A. N/A. |
| ME Intel Management Engine Interface | Windows 7. Windows 7 64-bit. | 7.0.0.1144. 7.0.0.1144. | N/A. N/A. |
| IRST | Windows 7. Windows 7 64-bit. | 10.0.0.1008. 10.0.0.1008. | N/A. N/A. |

### Additional Devices and Driver for Windows 7 32-bit and 64-bit

| Technology | OS | Version | Package version |
| --- | --- | --- | --- |
| TV Card Hauppauge Win TV HVR-1200 PCI-E x1 card | Windows 7. Windows 7 64-bit. | 1.44.27253.0. 1.44.27253.0. | N/A. N/A. |

## Windows Logo Kits Used (WLK)

Microsoft website: http://www.microsoft.com/whdc/DevTools/WDK/DTM.mspx

Please check regularly for test kit updates from Microsoft. Please ensure latest filters updated prior to WHQL run.

| Operating Systems | Notes | WHQL Testkit |
| --- | --- | --- |
| Windows 7 Windows 7 64-bit | WLK1.5 for Windows 7 | WLK1.5 for Windows 7 |

## Errata and Contingencies

| Operating System | Failing Test | Expiry Date | ID Number | Type | Error Description |
| --- | --- | --- | --- | --- | --- |
| Windows 7 | Class Driver AC3 Test - Win7 (System) | 06/01/2011 | 1890 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | Errata 1890 Timing issues in RadioOnOff Stress may result in the follwoing errors: Turning off the software radio reported success but activation state is ... , expected activation state is WwanActivationStateDeactivated Turning off the software radio reported success but PacketServiceState is ... , expected PacketServiceState is WwanPacketServiceStateDetached Turning off the software radio reported success but RegisterState is ... , expected RegisterState is WwanRegisterStateDeregistered |
| Windows 7 | Graphics HDMI System Test (Manual) | 06/01/2011 | 1717 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | GraphicsHDMITest: should ignore pin widgets that have "Port Connectivity" set to "No Physical Connection" . |
| Windows 7 | PCI Hardware Compliance Test For Systems | 06/01/2011 | 923 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | Assertion FAE18121-9177-4FB2-A081-0D04C285EFF2 Bit range 15:0 (Extended Capability ID)in the Enhanced Capability Header register (offset 0h) in the Unrecognized Enhanced Capability ID 13 table is Dh. It must be in the range [0x0 - 0xB] as all other Capability IDs are reserved. |
| Windows 7 | PCI Hardware Compliance Test For Systems | 12/01/2012 | 1241 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | This happens because the PCI Compliance test assumes that if the Data Link Layer Link Active Reporting Capable bit in the Link Capabilities register for a given PCIe port is set then that indicates that the Data Link Layer Link Active bit will also be set. This is an incorrect assumption because the Data Link Link Layer Link Active bit can be reset when there is no device below the port. This assertion needs to be removed from the PCIHCT. The current architecture of the PCIHCT prevents it from knowing whether devices exist below a bridge/port. |
| Windows 7 | PCI Hardware Compliance Test For Systems | 06/01/2011 | 401 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | The following PCI Compliance test failure is acceptable: Bit 15 (Bridge Configuration Retry Enable) in the Device Control register (offset 8h) in the PCI Express Capability table must be read-only and always return 0 as it is reserved for devices other than PCI Express to PCI/PCI-X Bridges. Assertion 13A41D3E-2576-41DC-A67C-525DA3637CEA This failure is acceptable because this is a PCIe 1.1 feature and the WLP requires compliance with only PCIe 1.0a. |
| Windows 7 | PCI Hardware Compliance Test For Systems | 12/31/2011 | 2051 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | PCIHCT - Bit range 11:10 (ASPM Support)in the Link Capabilities register (offset Ch) in the PCI Express Capability table is 0h. It cannot be in the set of values {0x0, 0x2}. (Assertion 5CAF4993-B8D1-4E4E-99EC-CC5895364E32) |
| Windows 7 | PCI Hardware Compliance Test For Systems | 06/01/2011 | 423 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | Assertion 5CAF4993-B8D1-4E4E-99EC-CC5895364E32 Bit range 11:10 (ASPM Support)in the Link Capabilities register (offset Ch) in the PCI Express Capability table is 0h. It cannot be in the set of values {0x0, 0x2}. |
| Windows 7 | UAA Test - Win7 (System) | 12/31/2011 | 1670 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | HDMI audio devices are backed by video pipes; if a graphics subsystem exposes three output ports, but only has two video pipes, it is not possible to get all three HDMI audio endpoints "active" simultaneously. |
| Windows 7 | UAA Test - Win7 (System) | 06/01/2011 | 1300 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | HD Audio pin configuration document calls out setting Port Connectivity to No Connection as the way to turn a pin off in a particular system. UAA Test incorrectly tests such pins. |
| Windows 7 | UAA Test - Win7 (System) | 07/01/2011 | 1466 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | Preview Filter: UAA Test - Intel Low Power DCN says "EPSS implies KeepAlive, but only after July 1st 2011" |
| Windows 7 | UAA Test - Win7 (System) | 02/28/2011 | 1299 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | Preview filter - Jack Detect Override on digital pin widgets Errata 1299 The HD Audio configuration default register (7.3.3.31 in the HD Audio specification) includes a "Jack Detect Override" flag that can be used to indicate that although a pin widget would normally be capable of jack detection, there is something about this particular system that causes this to be impossible. This was intended to be used, for example, for analog pin widgets that are connected to RCA jacks, which do not allow for impedance detection. Some digital pin widgets are using the Presence Detect pin sense response to indicate that a digital handshake has occured - indeed, HDMI pins have entire DCNs built around this concept, and it applies equally well to S/PDIF pins. A digital converter that supports presence detection should be able to do so in any system, so the "Jack Detect Override" concept should not apply to digital pins. |
| Windows 7 | UAA Test - Win7 (System) | 06/01/2015 | 513 | Erratum | Issue Description: |
| Windows 7 64-bit |  |  |  |  | UAA Test requires the Traffic Priority bit to be read/write - however there are two specs that apply, and they conflict. One says the bit must be read/write, the other says it must be read-only. Contact has been made with the author of both specs (Intel) but until this point is clarified we cannot fail submissions containing this test failure. |
| Windows 7 | Graphis HDMI System Test (Manual) | 12/31/2011 | 3272 | Filter | Issue Description: |
| Windows 7 64-bit |  |  |  |  | HDMI audio codecs sometimes expose more HDMI audio pins (audio outputs) than can be plugged in at any given time, due to resource costraints in the video hardware. Test will need to be rejiggered to allow testing each in turn. |

## Test Notes

| Operating System | Test | Description |
| --- | --- | --- |
| Windows 7 | BIOS download | Internal: http://bios.intel.com/downloads/ External: http://www.intel.com/ click on Support and Download |
| Windows 7 | BIOS setup | Please make sure the BIOS setting are as below, otherwise use default settings. System Date and Time: Current date and time Trusted Platform Module: Enable Others: BIOS default setting |
| Windows 7 filter update | WLK WHQL test | http://winqual.microsoft.com/member/SubmissionWizard/LegalExemptions/filterupdates.cab |
| Special H/W that use to PASS the test | None | None |

These are the product names to enter in the “Submission ID of previously logo’d qualified PC system or server” field during your “System Using a Previously Logo’d Motherboard” submission to Microsoft.